Cafe Butler Robot

ROS2 Developer Assessment - Technical Report

February 2026  |  ROS2 Humble  |  TurtleBot3 Burger

# 1. Introduction

This report documents the development of an autonomous butler robot for the French Door Cafe. The robot replaces human waitstaff by autonomously collecting food orders from the kitchen and delivering them to customer tables. The system is built entirely using ROS2 (Robot Operating System 2) with Nav2 for autonomous navigation and SLAM Toolbox for map generation.

The robot manages delivery for 3 tables (table1, table2, table3) and handles 7 different operational scenarios including timeouts, cancellations, and multiple simultaneous orders.

# 2. System Overview

## Robot Workflow

The general workflow of the robot is as follows:

- Robot starts at HOME position waiting for orders

- When an order is received, robot travels to KITCHEN to collect food

- Robot then travels to the CUSTOMER TABLE for delivery

- After delivery, robot returns to HOME position

## Technology Stack

| Component | Technology |
| --- | --- |
| Simulation | Gazebo Classic |
| Robot Model | TurtleBot3 Burger |
| Navigation | Nav2 (Navigation2) |
| Mapping | SLAM Toolbox |
| Localization | AMCL |
| Programming Language | Python 3 |
| ROS Version | ROS2 Humble |

# 3. Package Structure

| File / Folder | Purpose |
| --- | --- |
| robot_controller.py | Main brain - state machine and navigation logic |
| order_manager.py | Manages incoming order queue and dispatching |
| confirmation_node.py | Handles kitchen and table confirmations |
| launch/cafe.launch.py | Launches Gazebo + Nav2 + Robot together |
| worlds/cafe_world.world | Custom cafe Gazebo simulation world |
| maps/cafe_map.pgm + .yaml | Pre-built SLAM map of the cafe |
| config/nav2_params.yaml | Nav2 navigation parameters and tuning |

# 4. Robot State Machine

The robot uses a Finite State Machine (FSM) to manage its behavior. At any given moment, the robot is in exactly one of 7 states:

| State | Value | Description |
| --- | --- | --- |
| HOME | 0 | Robot is at home, waiting for orders |
| GO_TO_KITCHEN | 1 | Robot is navigating to kitchen |
| WAIT_KITCHEN_CONFIRM | 2 | At kitchen, waiting for food pickup confirmation |
| GO_TO_TABLE | 3 | Robot is navigating to customer table |
| WAIT_TABLE_CONFIRM | 4 | At table, waiting for delivery confirmation |
| RETURN_TO_KITCHEN | 5 | Returning to kitchen after failed table delivery |
| RETURN_HOME | 6 | Navigating back to home position |

## State Flow

HOME  ->  GO_TO_KITCHEN  ->  WAIT_KITCHEN_CONFIRM  ->  GO_TO_TABLE

->  WAIT_TABLE_CONFIRM  ->  RETURN_TO_KITCHEN  ->  RETURN_HOME  ->  HOME

# 5. Waypoint Locations

| Location | X | Y | Color in Gazebo | Role |
| --- | --- | --- | --- | --- |
| home | 0.0 | -1.0 | Blue Box | Robot starting position |
| kitchen | 1.5 | 0.0 | Pink/Red Box | Food pickup location |
| table1 | 2.0 | 2.0 | Green Box | Customer table 1 |
| table2 | -2.0 | 2.0 | Green Box | Customer table 2 |
| table3 | 0.0 | -2.0 | Green Box | Customer table 3 |

# 6. ROS2 Topics

| Topic | Type | Purpose |
| --- | --- | --- |
| /dispatch_order | std_msgs/String | Receive new order with table name |
| /cancel_order | std_msgs/String | Cancel an existing order |
| /kitchen_confirm | std_msgs/Bool | Kitchen confirms food is ready |
| /table_confirm | std_msgs/Bool | Table confirms food received |
| /navigate_to_pose | nav2_msgs/Action | Send navigation goal to Nav2 |

# 7. Milestones Implementation

| # | Milestone | Description | Status |
| --- | --- | --- | --- |
| 1 | Basic Delivery | Home -> Kitchen -> Table -> Home | Done |
| 2 | Timeout Handling | Returns home if no confirm in 8 seconds | Done |
| 3a | Kitchen Timeout | No kitchen confirm -> go home directly | Done |
| 3b | Table Timeout | No table confirm -> kitchen -> home | Done |
| 4 | Order Cancel | Smart return based on where robot is | Done |
| 5 | Multiple Orders | Deliver all tables then return home | Done |
| 6 | Skip Unconfirmed | Skip table with no confirm, kitchen last | Done |
| 7 | Cancel Specific | Skip canceled table, deliver remaining | Done |

# 8. How to Run

## Prerequisites

- Ubuntu 22.04 LTS

- ROS2 Humble

- TurtleBot3: ros-humble-turtlebot3, ros-humble-turtlebot3-gazebo

- Nav2: ros-humble-navigation2, ros-humble-nav2-bringup

- SLAM: ros-humble-slam-toolbox

## Installation

git clone <your-repo-url>

cd goat_robot_ws

source /opt/ros/humble/setup.bash

export TURTLEBOT3_MODEL=burger

colcon build && source install/setup.bash

## Launch Commands

Open 4 terminals and run one command in each:

# Terminal 1 - Launch simulation

ros2 launch cafe_butler_robot cafe.launch.py

# Terminal 2 - Launch RViz2

ros2 launch nav2_bringup rviz_launch.py

# Terminal 3 - Run robot controller

ros2 run cafe_butler_robot robot_controller

# Terminal 4 - Send order

ros2 topic pub --once /dispatch_order std_msgs/msg/String "{data: 'table1'}"

# 9. Conclusion

The Cafe Butler Robot successfully demonstrates autonomous food delivery in a simulated cafe environment. All 7 milestones have been implemented using core ROS2 concepts including topics, action clients, and state machines. The system is generic and easily extensible - adding more tables only requires adding entries to the locations dictionary.

The project uses SLAM Toolbox to build an accurate map of the cafe, and Nav2 to autonomously navigate between all 5 waypoints. The state machine ensures the robot handles all edge cases including timeouts, cancellations, and multiple orders cleanly.

Built for GOAT Robotics - ROS Developer Assessment